B1528 SIROCCO · Issue #598

# B1528 SIROCCO current-heading/rudder force comparison — 30 deg rudder limit

B1528 SIROCCO · Issue #598

This interactive report visualizes rudder-induced , OCIMF-inspired hull-current review , and summed total force/yaw-moment components for B1528 SIROCCO current-heading/rudder combinations with rudder angles limited to -30 deg through +30 deg. Totals are comparison loads only; this is not a validated whole-vessel current-load, mooring, tug, bank-effect, or compliance model.

Native Word version: this DOCX was rebuilt with editable Word paragraphs and tables. It is not a PDF page-image conversion; the schematic and chart data are included as editable tabular slices, and all HTML report visuals are embedded as static images for 1:1 review against the HTML output.

## Input data

Key input values used by both the interactive HTML and print/PDF report views.

| Field | Value |
| --- | --- |
| LBP | 225.500 m |
| Beam | 32.260 m |
| Draft | 12.200 m |
| Current frontal projected area | 393.572 m² |
| Current lateral projected area | 2751.100 m² |
| Yaw lever | 135.300 m |
| Rudder area | 44.939563 m² |
| Rudder span | 9.000 m |
| Ship speed over ground | 0.000 kn |
| Water density | 1025.0 kg/m³ |
| Barrass/workbook β | 600.0 |
| Propeller rotation factor Cr | 1.0 |
| Current speed sweep | 1, 1.5, 2, 2.5, 3, 3.5, 4, 4.5 kn + chart-default 4.56 kn |
| Heading/rudder grid | -10..10 deg heading × -30..30 deg rudder, 1 deg step |

## Sample calculation

Representative hand-check point: chart-default 4.56 kn, heading 0 deg, rudder +1 deg, Cr=1.0 .

V = 4.56 kn × 0.51444 = 2.34585 m/s ; displayed as 4.56 kn = 2.34585 m/s .

alpha = δ - ψ = 1.0 deg - 0.0 deg = 1.0 deg .

F = 600.0 × 44.939563 × 2.34585² × 1.0 = 148381.324 N .

Fn = F × sin(alpha) = 2589.611 N .

X_ship = 45.195 N , Y_ship = 2589.217 N , N_ship = 350.321028 kN-m .

q = 0.5 × rho × V² ; current coefficients use Cx=1.05×abs(cos(psi)) , Cy=sin(psi) , Cm=0.55×sin(psi) as transparent placeholder heading functions.

X_current = 1165484.513 N , Y_current = 0.000 N , N_current = 0.000000 kN-m ; totals are component sums at COG.

## Ship/current/rudder geometry schematic — editable description

Schematic image — ship/current/rudder geometry

Source: interactive HTML report schematic capture; adjacent editable bullets/table preserve the geometry convention and vector meanings.

This schematic updates with the current speed/rudder controls and uses the selected breakdown heading. It shows ship-fixed axes, the current heading offset ψ , rudder command δ , and effective rudder inflow angle alpha = δ - ψ . Positive bow-to-port convention applies to ψ , δ , Y_ship , and yaw moment.

Black dashed axes: ship-fixed COG frame; +X forward and +Y port.

Blue vector: current heading offset ψ relative to ship centerline.

Orange vector: selected rudder command δ ; the selector intentionally controls this angle for Chart 1/3/4 review.

Green vector: selected-case total horizontal force direction from current + rudder components.

Effective inflow: alpha = δ - ψ ; when alpha is zero, rudder-induced load is zero but hull-current load remains.

| Vector / convention | Meaning |
| --- | --- |
| +X ship / bow | Ship-fixed longitudinal axis through COG; positive forward. |
| +Y ship / port | Ship-fixed transverse axis through COG; positive port. |
| ψ current heading offset | Positive bow-to-port current-heading convention. |
| δ rudder command | Selected rudder angle, limited to -30 deg through +30 deg. |
| alpha = δ - ψ | Effective rudder inflow angle used for rudder-induced load. |
| Total force resultant | Vector sum of current and rudder X/Y components at COG. |

## Selected-speed envelope summary — 4.56 kn chart default

Envelope values use the full heading × rudder grid at 4.56 kn, preserving the chart-default extra speed case used in the HTML/PDF report.

| Metric | Envelope case | Value |
| --- | --- | --- |
| Selected speed | chart-default extra plane | 4.56 kn · 1281 rows |
| Max ¦Y_ship¦ at selected speed | heading -4 deg · rudder 30 deg | 65.384 kN |
| Max ¦N_ship¦ at selected speed | heading -4 deg · rudder 30 deg | 8846.487 kN-m |
| Max resultant horizontal force | heading -10 deg · rudder 30 deg | 95.378 kN |

## Chart 1 data — rudder-induced ship-fixed force components

Editable data slice corresponding to HTML default current 4.56 kn and rudder 0.0 deg.

### Chart 1 image — rudder-induced ship-fixed force components

Source: interactive HTML report chart capture; adjacent editable table preserves underlying values for comments.

| Heading ψ (deg) | Alpha (deg) | X_ship (kN) | Y_ship port (kN) | Resultant H (kN) | N_ship (kN-m) |
| --- | --- | --- | --- | --- | --- |
| -10 | 10 | 8.813 | 24.212 | 25.766 | 3275.918 |
| -9 | 9 | 7.173 | 22.076 | 23.212 | 2986.866 |
| -8 | 8 | 5.692 | 19.851 | 20.651 | 2685.802 |
| -7 | 7 | 4.375 | 17.546 | 18.083 | 2373.972 |
| -6 | 6 | 3.225 | 15.171 | 15.510 | 2052.655 |
| -5 | 5 | 2.246 | 12.736 | 12.932 | 1723.156 |
| -4 | 4 | 1.441 | 10.250 | 10.351 | 1386.802 |
| -3 | 3 | 0.812 | 7.723 | 7.766 | 1044.940 |
| -2 | 2 | 0.361 | 5.166 | 5.178 | 698.935 |
| -1 | 1 | 0.090 | 2.588 | 2.590 | 350.161 |
| 0 | 0 | 0.000 | 0.000 | 0.000 | 0.000 |
| 1 | -1 | 0.090 | -2.588 | 2.590 | -350.161 |
| 2 | -2 | 0.361 | -5.166 | 5.178 | -698.935 |
| 3 | -3 | 0.812 | -7.723 | 7.766 | -1044.940 |
| 4 | -4 | 1.441 | -10.250 | 10.351 | -1386.802 |
| 5 | -5 | 2.246 | -12.736 | 12.932 | -1723.156 |
| 6 | -6 | 3.225 | -15.171 | 15.510 | -2052.655 |
| 7 | -7 | 4.375 | -17.546 | 18.083 | -2373.972 |
| 8 | -8 | 5.692 | -19.851 | 20.651 | -2685.802 |
| 9 | -9 | 7.173 | -22.076 | 23.212 | -2986.866 |
| 10 | -10 | 8.813 | -24.212 | 25.766 | -3275.918 |

## Chart 2 data — yaw moment heatmap envelope by rudder angle

Editable table gives max absolute rudder-induced yaw moment at 4.56 kn for each rudder command across headings.

### Chart 2 image — yaw moment heatmap envelope by rudder angle

Source: interactive HTML report chart capture; adjacent editable table preserves underlying values for comments.

| Rudder δ (deg) | Heading at max ¦N¦ (deg) | N_ship (kN-m) | Y_ship port (kN) |
| --- | --- | --- | --- |
| -30 | 4 | -8846.487 | -65.384 |
| -29 | 4 | -8732.416 | -64.541 |
| -28 | 5 | -8616.243 | -63.683 |
| -27 | 6 | -8497.446 | -62.804 |
| -26 | 6 | -8383.375 | -61.961 |
| -25 | 7 | -8267.788 | -61.107 |
| -24 | 8 | -8149.683 | -60.234 |
| -23 | 8 | -8035.612 | -59.391 |
| -22 | 9 | -7920.824 | -58.543 |
| -21 | 10 | -7803.622 | -57.676 |
| -20 | 10 | -7689.551 | -56.833 |
| -19 | 10 | -7563.989 | -55.905 |
| -18 | 10 | -7427.086 | -54.893 |
| -17 | 10 | -7279.012 | -53.799 |
| -16 | 10 | -7119.945 | -52.623 |
| -15 | 10 | -6950.080 | -51.368 |
| -14 | 10 | -6769.624 | -50.034 |
| -13 | 10 | -6578.796 | -48.624 |
| -12 | 10 | -6377.830 | -47.138 |
| -11 | 10 | -6166.969 | -45.580 |
| -10 | 10 | -5946.472 | -43.950 |
| -9 | 10 | -5716.605 | -42.251 |
| -8 | 10 | -5477.651 | -40.485 |
| -7 | 10 | -5229.899 | -38.654 |
| -6 | 10 | -4973.651 | -36.760 |
| -5 | 10 | -4709.220 | -34.806 |
| -4 | 10 | -4436.928 | -32.793 |
| -3 | 10 | -4157.107 | -30.725 |
| -2 | 10 | -3870.097 | -28.604 |
| -1 | 10 | -3576.248 | -26.432 |
| 0 | -10 | 3275.918 | 24.212 |
| 1 | -10 | 3576.248 | 26.432 |
| 2 | -10 | 3870.097 | 28.604 |
| 3 | -10 | 4157.107 | 30.725 |
| 4 | -10 | 4436.928 | 32.793 |
| 5 | -10 | 4709.220 | 34.806 |
| 6 | -10 | 4973.651 | 36.760 |
| 7 | -10 | 5229.899 | 38.654 |
| 8 | -10 | 5477.651 | 40.485 |
| 9 | -10 | 5716.605 | 42.251 |
| 10 | -10 | 5946.472 | 43.950 |
| 11 | -10 | 6166.969 | 45.580 |
| 12 | -10 | 6377.830 | 47.138 |
| 13 | -10 | 6578.796 | 48.624 |
| 14 | -10 | 6769.624 | 50.034 |
| 15 | -10 | 6950.080 | 51.368 |
| 16 | -10 | 7119.945 | 52.623 |
| 17 | -10 | 7279.012 | 53.799 |
| 18 | -10 | 7427.086 | 54.893 |
| 19 | -10 | 7563.989 | 55.905 |
| 20 | -10 | 7689.551 | 56.833 |
| 21 | -10 | 7803.622 | 57.676 |
| 22 | -9 | 7920.824 | 58.543 |
| 23 | -8 | 8035.612 | 59.391 |
| 24 | -8 | 8149.683 | 60.234 |
| 25 | -7 | 8267.788 | 61.107 |
| 26 | -6 | 8383.375 | 61.961 |
| 27 | -6 | 8497.446 | 62.804 |
| 28 | -5 | 8616.243 | 63.683 |
| 29 | -4 | 8732.416 | 64.541 |
| 30 | -4 | 8846.487 | 65.384 |

## Chart 3 data — OCIMF current vs rudder vs total horizontal resultant

Editable default trace: current 4.56 kn, rudder 0.0 deg. Current coefficients are transparent placeholder heading functions, not vessel-specific OCIMF curves.

### Chart 3 image — OCIMF current vs rudder vs total horizontal resultant

Source: interactive HTML report chart capture; adjacent editable table preserves underlying values for comments.

| Heading ψ (deg) | OCIMF current R (kN) | Rudder R (kN) | Total R (kN) | Total X (kN) | Total Y port (kN) |
| --- | --- | --- | --- | --- | --- |
| -10 | 1769.931 | 25.766 | 1757.358 | 1156.591 | -1323.104 |
| -9 | 1672.818 | 23.212 | 1661.861 | 1158.308 | -1191.681 |
| -8 | 1580.529 | 20.651 | 1571.231 | 1159.834 | -1059.978 |
| -7 | 1494.083 | 18.083 | 1486.455 | 1161.172 | -928.024 |
| -6 | 1414.664 | 15.510 | 1408.681 | 1162.325 | -795.853 |
| -5 | 1343.624 | 12.932 | 1339.210 | 1163.295 | -663.496 |
| -4 | 1282.450 | 10.351 | 1279.468 | 1164.086 | -530.983 |
| -3 | 1232.690 | 7.766 | 1230.936 | 1164.699 | -398.346 |
| -2 | 1195.835 | 5.178 | 1195.028 | 1165.136 | -265.615 |
| -1 | 1173.148 | 2.590 | 1172.942 | 1165.397 | -132.823 |
| 0 | 1165.485 | 0.000 | 1165.485 | 1165.485 | 0.000 |
| 1 | 1173.148 | 2.590 | 1172.942 | 1165.397 | 132.823 |
| 2 | 1195.835 | 5.178 | 1195.028 | 1165.136 | 265.615 |
| 3 | 1232.690 | 7.766 | 1230.936 | 1164.699 | 398.346 |
| 4 | 1282.450 | 10.351 | 1279.468 | 1164.086 | 530.983 |
| 5 | 1343.624 | 12.932 | 1339.210 | 1163.295 | 663.496 |
| 6 | 1414.664 | 15.510 | 1408.681 | 1162.325 | 795.853 |
| 7 | 1494.083 | 18.083 | 1486.455 | 1161.172 | 928.024 |
| 8 | 1580.529 | 20.651 | 1571.231 | 1159.834 | 1059.978 |
| 9 | 1672.818 | 23.212 | 1661.861 | 1158.308 | 1191.681 |
| 10 | 1769.931 | 25.766 | 1757.358 | 1156.591 | 1323.104 |

## Chart 4 data — OCIMF current vs rudder vs total yaw moment about COG

Editable default trace: current 4.56 kn, rudder 0.0 deg. Positive yaw moment is bow-to-port.

### Chart 4 image — OCIMF current vs rudder vs total yaw moment about COG

Source: interactive HTML report chart capture; adjacent editable table preserves underlying values for comments.

| Heading ψ (deg) | OCIMF current N (kN-m) | Rudder N (kN-m) | Total N (kN-m) |
| --- | --- | --- | --- |
| -10 | -167100.926 | 3275.918 | -163825.008 |
| -9 | -150536.241 | 2986.866 | -147549.375 |
| -8 | -133925.702 | 2685.802 | -131239.900 |
| -7 | -117274.367 | 2373.972 | -114900.395 |
| -6 | -100587.310 | 2052.655 | -98534.654 |
| -5 | -83869.612 | 1723.156 | -82146.457 |
| -4 | -67126.367 | 1386.802 | -65739.566 |
| -3 | -50362.675 | 1044.940 | -49317.735 |
| -2 | -33583.642 | 698.935 | -32884.707 |
| -1 | -16794.379 | 350.161 | -16444.218 |
| 0 | 0.000 | 0.000 | 0.000 |
| 1 | 16794.379 | -350.161 | 16444.218 |
| 2 | 33583.642 | -698.935 | 32884.707 |
| 3 | 50362.675 | -1044.940 | 49317.735 |
| 4 | 67126.367 | -1386.802 | 65739.566 |
| 5 | 83869.612 | -1723.156 | 82146.457 |
| 6 | 100587.310 | -2052.655 | 98534.654 |
| 7 | 117274.367 | -2373.972 | 114900.395 |
| 8 | 133925.702 | -2685.802 | 131239.900 |
| 9 | 150536.241 | -2986.866 | 147549.375 |
| 10 | 167100.926 | -3275.918 | 163825.008 |

## Selected-case force breakdown

Default-trace breakdown at 4.56 kn, rudder 0 deg, heading -10 deg (maximum absolute total yaw moment for selected trace).

| Component | X ship (kN) | Y ship port (kN) | Horizontal resultant (kN) | Yaw moment about COG (kN-m) |
| --- | --- | --- | --- | --- |
| OCIMF current | 1147.778 | -1347.316 | 1769.931 | -167100.926 |
| Rudder induced | 8.813 | 24.212 | 25.766 | 3275.918 |
| Total / mooring reaction opposite | 1156.591 | -1323.104 | 1757.358 | -163825.008 |

## Method and provenance

V=kn*0.51444; F=beta*A_R*V^2*Cr; alpha=delta-psi; X_local=F*sin(alpha)^2; Y_local=F*sin(alpha)*cos(alpha); X_ship=X_local*cos(psi)-Y_local*sin(psi); Y_ship=X_local*sin(psi)+Y_local*cos(psi); N=Y_ship*yaw_lever

Heading/rudder effective-angle convention: alpha = rudder_angle_deg - heading_offset_deg.

Local-to-ship transform rotates local current-frame X/Y loads into ship-fixed COG axes.

Current review equations: q=0.5*rho*V²; Cx=1.05*abs(cos(psi)); Cy=sin(psi); Cm=0.55*sin(psi); X_current=q*(beam*draft)*Cx; Y_current=q*(LBP*draft)*Cy; N_current=q*(LBP*draft)*LBP*Cm.

When rudder_angle_deg equals heading_offset_deg, alpha is zero and this rudder-induced component is zero; that is not total hull current load.

Scope note: hull current loads are first-cut OCIMF-inspired comparison loads using placeholder heading functions only; this is not a validated whole-vessel current-load or oblique-current hull/rudder interaction model.

## Summary

| Field | Value |
| --- | --- |
| row_count | 11529 |
| requested_engineering_row_count | 10248 |
| extra_default_row_count | 1281 |
| max_abs_ship_sway_force_N | 65384.23353202306 |
| max_abs_ship_sway_signed_force_N | 65384.23353202306 |
| max_abs_ship_sway_case | current 4.56 kn, heading -4 deg, rudder 30 deg |
| max_abs_yaw_moment_kN_m | 8846.48679688272 |
| max_abs_yaw_signed_moment_kN_m | 8846.48679688272 |
| max_abs_yaw_case | current 4.56 kn, heading -4 deg, rudder 30 deg |
| max_resultant_horizontal_force_N | 95377.67650976167 |
| max_resultant_case | current 4.56 kn, heading -10 deg, rudder 30 deg |
| max_ocimf_current_resultant_horizontal_force_N | 1769931.1993048876 |
| max_ocimf_current_resultant_case | current 4.56 kn, heading -10 deg, rudder -30 deg |
| max_total_resultant_horizontal_force_N | 1813872.9536408707 |
| max_total_resultant_case | current 4.56 kn, heading -10 deg, rudder -30 deg |
| max_abs_total_yaw_moment_kN_m | 173863.00417282406 |
| max_abs_total_yaw_signed_moment_kN_m | -173863.00417282406 |
| max_abs_total_yaw_case | current 4.56 kn, heading -10 deg, rudder -30 deg |

Source artifact: b1528_sirocco_current_heading_rudder_30deg_limit_report.html

Replacement note: native DOCX generated directly from HTML-embedded report data and static report text; the schematic and chart images were captured from the interactive HTML report and embedded as static review visuals, while adjacent Word tables preserve editable values.